M/V “ARBOL GRANDE”

MAIN PARTICULARS

| # Name | ARBOL GRANDE |
| --- | --- |
| # Owner | NORTH SEA COMMANDER SHIPPING AS through NAVIERA ZAMBRA A.I.E. |
| # Operator | OCEANTEAM - DIAVAZ |
| # Management | NORTH SEA SHIPPING AS |
| # Type of contract | 5 years charter |
| Type | IMR / Research vessel |
| Built | 2002 – 03 / Metalships & Docks - Vigo - Spain |
| Keel laid - Launched | 28.02.2002 – 18.04.2002 |
| Class. S. | DNV |
| DNV id. N. | D23411 |
| Class | +1A1, SF, HELDK-SH, DYNPOS-AUTR, DK(+), E0 |
| ERN | 99.0, 99.0, 98.5 |
| Call Sign | E C A O |
| Flag | Spain (Canary Islands) |
| Port of Registry | Santa Cruz de Tenerife |
| Spanish Reg. N. | 15 / 2002 |
| IMO number | 9264867 |
| LOA | 94.28 m. / 309.3 f. |
| LBP | 79.90 m. / 262.1 f. |
| Freeboard | 2512 mm / 8.24 f. (minimum) |
| Max draft | 7.0 m. / 23 f. |
| Displacement Max. D. | 7400 MT |
| H. from BL to Masthead | 41.5 m. |
| Beam | 20.00 m. / 65.6 f. |
| GT | 5530,25 |
| NT | 1659,07 |
| DW | 3996 MT |
| Depth moulded M. deck | 9.5 m |
| Depth moulded Twin d. | 6.5 m |
| Total Power | 8680 Kw |
| Cruising speed | 15.0 Kn. |
| Max. speed | 16.0 Kn. |
| Cruising period | 65 – 70 days |
| ##### COMMUNICATIONS |  |
| Inmarsat M - Voice | 7 636 319 75 / 76 |
| “ Fax | 7 636 319 77 |
| “ Data | 7 636 319 78 |
| Inmarsat B - Voice | 3 224 240 10 / 11 / 12 / 13 |
| “ Fax | 3 224 240 14 / 15 |
| “ ASD Data | 3 224 240 16 |
| “ HSD Data | 3 910 349 79 |
| Stratos Voice | + (1) 8324618029 |
| Mobile ph. Mexico | + (52) 9383890104 |

ENGINE & PROPULSION

| # Main engine | 2 x Wartsila 8L26 A, 2480 Kw each |
| --- | --- |
|  | 2 x Wartsila 6L26 A, 1860 Kw each |
| Main generators | 2 x Alconza NIR5680A-8LW, 2208 Kva |
|  | 2 x Alconza NIR6378A-8LW, 2967 Kva |
| Main propeller | 2 x Aquamaster US-255, 2400 Kw each |
| Fwd Tun. Thrusters | 2 x Kamewa Ulstein 250-TVSS, 883 Kw each |
| Fwd Azim. Thruster | 1 x Ulstein Aquamaster TCNS-83/56-220, 1200 Kw |
| Emergency Engine | 1 x Volvo Penta TAMD-163A, 428 Kw |
| Emergency Generator | 1 x Stamford MHC 534C1, 450 Kva |

ENGINE ROOM EQUIPMENT

| # Electronic | Aker IAS system, Automation and Monitoring System (+PMS) |
| --- | --- |
| Tank Sounding | Rolls Royce |
| Boiler | Pyro 406 Kw |
| Incinerator | Detegasa, Delta type IRL-50, 500000 Kcal / h. Oil residues capacity = 80 L/h |
| Fresh Water Gen. | 3 x two stage Aquamar, Type AQ-25/2, 24 m3 / day each |
| Start air Compress. | 2 x Sperre, 50 m3 / h each |
| Service air | 1 x Tamrotor 315 m3 / h |
| OWS | 1 x Detegasa. 15 ppm. Approved according res. MEPC.60 |
| Pumps | AZCUE |
| Hydrophore system | 2 x 15 m3 / h pumps plus 1000 liters tank |
| Sewage system | Type DVZ-SKA-150 “BIOMASTER” and jets vacuum system |

## Capacities & Consumption

| # Fuel capacity | 1342 m3 |
| --- | --- |
| Fuel consumption | Approx. 26 m3/day in transit – 10 m3/day DP Operation |
| Lube oil capacity | 33 m3 |
| Lube oil consumption | Approx. 60 L/day DP Operation |
| Fresh water capacity | 411 m3 |
| Fresh water cons. | Variable according staff and working area |
| Ballast capacity | 2460 m3 |
| Oily waters capacity | 35.72 m3 (Tanks 46, 47, 49) |
| Hold N 1 | 817 m3 |
| Hold N 2 | 330 m3 |

## Positioning System

| # DP System | Kongsberg Simrad SDP 21 |
| --- | --- |
| Windsensors | 2 x Windsensors WindObserver II |
| MRU | 2 x MRU-2 1 x MRU-5 |
| Hipap | Kongsberg Simrad 500 system |
| Fanbeam | MDL Fanbeam laser MK4 w / auto tilt |
| Taut Wire | BANDAK AS - Type LTW MK12 |
| GPS receiver | 2 x DARPS SEATEX |
| Spotbeam | Spotbeam receiver & demodulator |
| Inmarsat B | 1 Demodulator |
| Gyro | 3 x C. Plath Navigat X MK1, Mod 10 |
| DP Printer | 1 x Matrix Printer w / cable RS 232-C interface |
| UPS | 2 x UPS POWEC 2KVA (30 min) |
| Independent Joystick System | Kongsberg Simrad Type SJS 01 |

## GMDSS

| MMSI Number | 224 240 000 |
| --- | --- |
| # Maritime areas authorised | A1, A2, A3 |
| # HF/MF Radio Station | Sailor HF/MF P4000 / 500W Serial 4160170008 Homolog. N. 44.0010 |
| # VHF / DSC | 2 x Sailor RT-4722 Serial 3171010003 / 4 Homolog. N. 51.0013 |
| EPIRB | Jotron TRON 40S Serial 130AD12202 Homol. 65.0012 406.025 / 121.5 MHz |
| Radar Transponder | 2 x Jotron |
| Inmarsat C | 2 x Sailor H-2095-C Serial 2215518 / 2214158 Homol. 81.0018 1626.5 – 1645.5 MHz |
| Navtex | JRC NCR-330 Serial GD62111 Homol. 25.0014 |
| VHF Transceivers | 2 x Sailor RT-4800 Serial 3172360002 / 3 Homol. 50.0085 |
| VHF Portable | 3 x Jotron TRON TR20 GMDSS Serial 1114 / 1205 / 1242 Homol. 53.0019 |
| A.I.S. | Leica MX-420 Serial 800508 Homol. 56.0001 |
| Portable UHF | 6 x Motorola |
| Helicopter VHF com. | 2 x Jotron TRON-AIR Serial 958 / 959 Homol. 55.0001 121.5 / 123.1 MHz |
| Helicopter Beacon | Southern Navionic |
| Inmarsat B | Nera SATURN BM MK2 Serial 3EB002ECFFD5 Homol. 83.0011 1626.5 – 1645.5 MHz |
| SART | 2 x Jotron TRON SART Serial 32353 / 32354 Homol. 87.0002 9200 – 9500 MHz |

## Electronics

| # Autopilot | Robertson MK9 mark 3 |
| --- | --- |
| Radar ARPA X | JRC JMA-9822-6XA Serial LW22.163 Homol. 85.0282 9410 MHz |
| Radar ARPA S | JRC JMA-9832-SA Serial LW22.255 Homol. 85.0280 3050 MHz |
| GPS | Leica MX420 GPS Serial 801078 Homol. 97.0009 |
| Electronic Chart Sys. | Furuno |
| Doppler Speed Log | Sal |
| Echo Sounder | Skipper GDS 101 Serial 21845 Homol. 2.0073 |

Deck Space / Loading Details

| # Deck Space available | 940 m2 |
| --- | --- |
| Deck Length | 47 m |
| Deck Strength | 5 MT / m2 |
| Max. Cargo Capacity | Approx. 1600 MT |
| Description | Wooden deck Fr. 10 to Fr. 70 with transverse “T” bars every second frame. “T” bars: Web 65 x 15, Fl. 150 x 15 Securing of Equipment: by use of brackets welded to transverse “T” bars. |
| Working moonpool | 7,2 x 7,2 m. For module handling (light opening toward the sea) Flush hatch on main deck, hydraulic operated. |

## Deck Equipment Permanently Installed

| # Main Crane | Norlift-Hydraulic Offshore GPCTO 2000-6041 Crane. |  |  |
| --- | --- | --- | --- |
|  | Main Single | Main Double | Aux Single |
| Max lifting Capacity. Platform lift | - | - | 15 T. |
| Max lifting Capacity. Sea lift | 30 / 25 T * | 60 / 25 T * | 15 T * |
| Max outreach telescope in | 20 / 25 m | 14 / 25 m | 26 m |
| Max outreach telescope out | - | - | 41 m |
| Min outreach | 5 m | 5 m | 5.5 m |
| Hook travel max. | 350 m | 175 m | 150 m |
| Hook speed light load m/min | 60 | 30 | 60 |
| Hook speed full load m/min | 30 | 15 | 60 |
| Sleeving sector |  | 360 deg |  |
| Luffing time (average up/down) |  | 70 sec. |  |
| Sleeving speed |  | 0.8 RPM |  |
| List / Trim |  | 5 / 2 deg |  |
| * Weight of wire rope will reduce the SWL during subsea operation |  |  |  |
| # Service Crane Fr.70 | Norlift 2 T / 16 m. Hydralift KMCV 1301-2-16 |  |  |
| # Provision Crane | Norlift 2 T / 8 m |  |  |
| # Capstan | 1 each side aft main deck. Sec Ten Horn EHCV-50 kn Speed 10 m/min |  |  |
| # Anchor/Mooring winch | 2 fore deck. Sec Ten Horn EHAMW-50K3-80 kn Chain speed = 9m/min. Rope speed = 18 m/min Max. chain diameter = 50 mm |  |  |
| # ROV utilities | 2 x power outlets on main deck (440 V and 230 V) |  |  |
| # Helideck | For Super Puma MK II size helicopters. Hydro Aluminium. |  |  |
| # Anclas y cadenas |  |  |  |
| # Chains | 19 lengths x 27,5 m each. Total length = 522.5 m. Total mass = 28751.4 Kg. 20 Un. Kenter Total = 380 Kg. 2 End shackles Total = 90.4 Kg. 2 Swivels Total = 192 Kg. |  |  |
| # Anchors | Length of shank = 2319 mm. Flukes span = 2100 mm. Total mass = 3100 Kg. |  |  |

## Rescue & Safety Equipment

| # Lifeboat | 4 x 50 persons. Umoe Schat-Harding AS. Type KISS700 App. weight of boat fully equiped = 2900 Kg. App. weight of boat fully manned = 6650 Kg. Engine: 3 cylinder in line, four stroke diesel. 20.6 Kw / 2800 rpm |
| --- | --- |
| Rescue boat | 1 x 6 persons Waterjet |
| Single Point Davit | Hydramarine |
| Life Rafts | 8 x 25 persons |
| Immersion Suits | 6 |
| Lifejackets | 224 |

## Accommodation Details

| # Total Capacity | 200 Persons |
| --- | --- |
| # Maximum non Crew members capacity | 181 Persons (Acc. Safety Const. Cert.) |
| ##### Main Deck | 20 x 6 person cabins TOTAL = 120 person |
| 1st Acc Deck | 2 x 1 person cabins 22 x 2 person cabins 2 x 4 person cabins TOTAL = 54 person On / Offline room Conference Room |
|  | Hospital – 2 beds |
| 2nd Acc Deck | Mess Room Galley 2 x Day room 2 x Office |
| 3rd Acc Deck | 22 x 1 person cabins |
| 4th Acc Deck | 4 x 1 person cabins |
| ##### Tween Deck | Laundry, Drying, Gymnasium, 2 x Sauna |
| ##### Tween Deck Aft | Workshop, Deckstore, Paint Store |
| Tween Deck / Tank top | Workshop, Store, Electric Workshop |

Minimum Safe Manning

| #### For a number of persons on board below 30 |  |
| --- | --- |
| Captain | 1 |
| Chief Off | 1 |
| Deck Off | 1 |
| Chief Eng | 1 |
| 1st Assis. Eng. | 1 |
| 2nd Eng. | 1 |
| Electrician | 1 |
| A/B | 4 |
| Cook | 1 |
| ### Total | 12 |
| #### For a total of persons on board between 30 and 99 |  |
| Add to the above the following: |  |
| ### Cook | 1 |
| ### Messman | 1 |
| ### Total | 14 |
| #### For a total of persons on board above 99 |  |
| Add to the above the following: |  |
| ### Cook | 2 |
| ### Messman | 2 |
| ### Total | 18 |